| Car and engine specification |  |  |
| --- | --- | --- |
| BMW M5 F90 Sedan, 06/2018=> 4.4l V8 441 kW (S63B44B, with GPF, EURO 6d-TEMP) BMW M5 F90 Competition Sedan, 06/2018=> 4.4l V8 460 kW (S63B44B, with GPF, EURO 6d-TEMP) |  |  |
| INFOBOX: - Axle-back-system L/R: 089518 0500LR (incl. EC type approval) + tail pipe set! Original exhaust will need to be cut! No rear skirt modifications required! - RACING cat-back-system L/R: 089317 0000 (No EC type approval) + 089518 0500LR (incl. EC type approval) + tail pipe set! No cutting modifications required! When installing 089317 0000 the original BMW part „heat protection for tank“, oe-part-number 51 48 8 060 077 has to be used! When installing the optional non-resonated cat-back front section 089317 0000 in cars with GPF a deactivation oft he GPF in the engine control unit (ECU remapping) must be done in order to prevent “check engine light” warning signal and also to achieve optimized performance and throttle response. - The use of racing products and products which do not have EC approval is generally not permitted on public roads. - When installing REMUS axle-back and cat-back or catalyst delete tubes on vehicles which are used for closed road driving, it is advisable to tune the ECU for a maximum gain in performance and to ensure that the CEL does not come on. |  |  |
| Part no. | Description | € RRP price excl. VAT |
| 089317 0000 | Non-resonated cat-back front section, can only be installed in combination with the REMUS axle-back sport exhaust system Original tube Ø 80 mm - REMUS tube Ø 84 mm No EC type approval For mounting of 089317 0000 the original BMW part „heat protection for tank“, oe-part-number 51 48 8 060 077 has to be used! When fitting 089317 0000 on cars with GPF a deactivation in the engine control unit (ECU remapping) has to be done. | 650,-- |
| 089518 0500LR | Axle-back sport exhaust system, stainless steel, with integrated valves Original tube Ø 80 mm - REMUS tube Ø 84 mm incl. EC type approval The activation of the valves is carried out using the original actuators via the vehicle onboard electronics.. | 1.470,-- |
| Choose from following tail pipe package: |  |  |
| 0446 70CS | Carbon tail pipe set 4 tail pipes Ø 102 mm angled (shorter length 145 mm), Titanium internals, with adjustable spherical clamp connection | 876,-- |
| 0446 70SG | Stainless steel tail pipe set 4 tail pipes Ø 102 mm angled (shorter length 145 mm), straight cut, chromed, with adjustable spherical clamp connection | 308,-- |